Home Oxygen Monitoring

Patient Instructions

How to use oxygen monitor

- Insert two AAA batteries.

  - Push the battery cover horizontally along the arrow shown on the back panel

  - Install two AAA batteries

  - Close battery cover

- If any finger nail polish, remove before use.

- Open the monitor and insert pointer finger into the rubber hole.

  - Monitor should face up

  - Your fingernail should face up

- Rest hand on table with oxygen monitor on finger.

- Press button on monitor to turn on.

- Read numbers on screen after at least one minute of rest.

  - Left number is oxygen saturation (%SpO2)

  - Right number is heart rate* (beats per minute)

*We are not focusing on heart rate

When to use oxygen monitor

- Record oxygen measurement three times per day.

  - Morning, Midday, and Evening.

  - NOT overnight

  - Record measurements on second page

- And measure oxygen when you feel the following symptoms:

  - Difficulty Breathing

  - Dizziness

  - Confusion

  - Worsening Fatigue

When to contact respiratory clinic

- 93 - 100% SpO2: Oxygen levels are good. No call needed unless you have new symptoms or worsening shortness of breath.

- Less than 93%: Oxygen levels may be low.

  - Ensure you are seated with hand at rest on table

  - Try monitor on another finger for a few minutes at rest

  - If still less than 93%, call clinic at 617-284-7000

| Date | Time | Oxygen (%SpO2) |
| --- | --- | --- |